Name: Meghaj Nandumar BarhateProgramme: M.Tech. in Fluid and Thermal SciencesRoll Number: 261050047

# Rectangular Plate: Steady-State Heat Conduction

A thin rectangular plate of width $a$ and height $b$ has three sides held at $0^\circ\mathrm{C}$ while the fourth (top) side is held at a uniform temperature $T_0$. There is no internal heat generation and the plate is thin enough that conduction may be treated as two-dimensional, so that the steady-state temperature field satisfies Laplace's equation
$$
\frac{\partial^2 T}{\partial x^2}
+
\frac{\partial^2 T}{\partial y^2}
=0,
\qquad
0 \le x \le a,
\qquad
0 \le y \le b.
$$

#### Boundary Conditions

$$
T(x,0)=0,
\qquad
T(0,y)=0,
\qquad
T(a,y)=0,
\qquad
T(x,b)=T_0.
$$

#### Data

$$
a=0.4\ \mathrm{m},
\qquad
b=0.2\ \mathrm{m},
\qquad
T_0=100^\circ\mathrm{C}.
$$

#### Analytical Solution

# By separation of variables,
$$
T(x,y)

\frac{4T_0}{\pi}
\sum_{n=1,3,5,\ldots}^{\infty}
\frac{1}{n}
\sin\left(\frac{n\pi x}{a}\right)
\frac{
\sinh\left(\frac{n\pi y}{a}\right)
}{
\sinh\left(\frac{n\pi b}{a}\right)
}.
$$
Use enough terms to ensure the series has converged to at least five significant figures before using it as your benchmark.

Fig. 1: Rectangular plate with three sides at $0^\circ\mathrm{C}$ and the top side at $T_0$; inset shows the interior 5-point stencil at node $(i,j)$.

## Tasks

## $$
[A]{T}={b}
$$
for a uniform grid with
$$
\Delta x = \Delta y = h.
$$

## $$
h = 0.02,; 0.01,; 0.005\ \mathrm{m}.
$$

• Jacobi

• Gauss-Seidel

• SOR
For SOR, try at least three values of the relaxation factor $\omega$, including the theoretical optimum for a rectangular Dirichlet problem.
Record the number of iterations and CPU time for each method.

$$
\left(\frac{a}{4},\frac{b}{2}\right),
\qquad
\left(\frac{a}{2},\frac{b}{2}\right),
\qquad
\left(\frac{3a}{4},\frac{b}{2}\right),
\qquad
\left(\frac{a}{2},\frac{3b}{4}\right).
$$

Tabulate the maximum absolute error versus $h$ and confirm the expected accuracy of the 5-point scheme.
$$
O(h^2)
$$

## Numerical Methods Project Requirements

#### 1. Discretization Derivation

Derive the finite-difference (or control-volume) discretization from first principles before writing any code.

#### 2. Analytical Validation

Wherever an analytical solution is given, validate your numerical solution against it at several interior points and report the maximum absolute and percentage errors, not just a visual comparison of plots.

#### 3. Linear System Solution

Solve the linear system $[A]{T} = {b}$ by:

• (i) Direct method: Gaussian elimination / LU decomposition, exploiting bandedness where possible.

• (ii) Iterative methods: Jacobi, Gauss–Seidel, and SOR, as specified in each problem.

For each method, report:

• Number of iterations

• Wall-clock time

• Memory footprint (as the grid is refined)

Then discuss which approach you would recommend and why.

#### 4. Grid-Refinement (Convergence) Study

Perform a grid-refinement study for every problem. Use at least three grid sizes, with error plotted or tabulated against $\Delta x$, $\Delta r$, or $\Delta \theta$ as appropriate.

#### 5. Convergence Criteria and Parameters

State all of the following clearly:

• Convergence criteria (e.g., $\max |T^{p+1} - T^p| < 10^{-6}$)

• Initial guesses used

• Relaxation factors used (for SOR or other iterative methods)

## Import Libraries

import matplotlib.pyplot as plt
import numpy as np
from scipy.linalg import lu_factor, lu_solve
import os
import time as t
%pip install -qq numpy scipy matplotlib psutil ipympl
%matplotlib widget
%matplotlib widget
from pathlib import Path
from matplotlib.cm import ScalarMappable
from matplotlib.colors import Normalize

Note: you may need to restart the kernel to use updated packages.

## Generate Problem Statement Schematic (Figure 1)

from matplotlib.patches import Rectangle
from matplotlib.lines import Line2D

      
# ------------------------------------------------------------
# Geometry
# ------------------------------------------------------------
length = 0.4
breadth = 0.2
#Placeholder for Graphical representation of the domain and the stencil points
dx = 0.02
dy = 0.02
y_shift = 0.1

      
#Problem has 1 Axis of Symmetry, so we can use half the domain for the simulation. The other half can be obtained by mirroring the results.
xmin, xmax = -length / 2, length / 2
ymin_ref, ymax_ref =0.0 , breadth
ymin, ymax = ymin_ref , ymax_ref

      
# ------------------------------------------------------------
# Mesh coordinates
# ------------------------------------------------------------
x_grid = np.arange(xmin, xmax + 1e-12, dx)
y_grid = np.arange(ymin_ref, ymax_ref + 1e-12, dy) + y_shift

      
# ------------------------------------------------------------
# Stencil points measured from the origin
# ------------------------------------------------------------
points = {
    "(i,j)": (3 * dx, 3 * dy + y_shift),     # red
    "(i,j-1)": (3 * dx, 2 * dy + y_shift),   # blue
    "(i,j+1)": (3 * dx, 4 * dy + y_shift),   # violet
    "(i-1,j)": (2 * dx, 3 * dy + y_shift),   # green
    "(i+1,j)": (4 * dx, 3 * dy + y_shift),   # purple
}

      
# ------------------------------------------------------------
# Location to move delta x and delta y arrows: grid point (8,3)
# ------------------------------------------------------------
base_x = 8 * dx
base_y = 3 * dy + y_shift

      
# ------------------------------------------------------------
# Plot
# ------------------------------------------------------------
fig, ax = plt.subplots(figsize=(10, 5))

      
# Faint light-grey dashed mesh
for x in x_grid:
    ax.plot([x, x], [ymin , ymax], linestyle="--", linewidth=0.7, color="lightgrey", alpha=0.8, zorder=1)

      
for y in y_grid:
    ax.plot([xmin, xmax], [y - y_shift, y - y_shift], linestyle="--", linewidth=0.7, color="lightgrey", alpha=0.8, zorder=1)

      
# Rectangle boundary in LaTeX blue
ax.add_patch(Rectangle((xmin, ymin), length, breadth, fill=False, edgecolor="#0000FF", linewidth=2.2, zorder=2))

      
# Cartesian axes through origin
ax.axhline(0, color="black", linewidth=0.9, zorder=0)
ax.axvline(0, color="Red", linewidth=0.9, zorder=0)

      
# ------------------------------------------------------------
# Stencil points
# ------------------------------------------------------------
ax.scatter(*points["(i,j)"], s=90, color="red", zorder=5)
ax.scatter(*points["(i,j-1)"], s=75, color="#0000FF", zorder=5)
ax.scatter(*points["(i,j+1)"], s=75, color="violet", zorder=5)
ax.scatter(*points["(i-1,j)"], s=75, color="green", zorder=5)
ax.scatter(*points["(i+1,j)"], s=75, color="purple", zorder=5)

      
# Mark the reference location (8,3)
ax.scatter(base_x, base_y, s=25, color="black", zorder=6)

      
# ------------------------------------------------------------
# Move delta x and delta y arrows to grid point (8,3)
# ------------------------------------------------------------
ax.annotate("", xy=(base_x + dx, base_y), xytext=(base_x, base_y), arrowprops=dict(arrowstyle="<->", color="black", linewidth=1.5), zorder=6)
ax.annotate("", xy=(base_x, base_y + dy), xytext=(base_x, base_y), arrowprops=dict(arrowstyle="<->", color="black", linewidth=1.5), zorder=6)
ax.text(base_x + dx / 2, base_y - 0.012, r"$\Delta x$", ha="center", va="top", fontsize=11)
ax.text(base_x - 0.012, base_y + dy / 2, r"$\Delta y$", ha="right", va="center", rotation=90, fontsize=11)

      
# ------------------------------------------------------------
# Index / legend for stencil-point colors
# ------------------------------------------------------------
legend_handles = [
    Line2D([0], [0], marker='o', linestyle='None', markerfacecolor='red', markeredgecolor='red', markersize=8, label=r"Red Point: $(i,j)$"),
    Line2D([0], [0], marker='o', linestyle='None', markerfacecolor='#0000FF', markeredgecolor='#0000FF', markersize=8, label=r"Blue Point: $(i,j-1)$"),
    Line2D([0], [0], marker='o', linestyle='None', markerfacecolor='violet', markeredgecolor='violet', markersize=8, label=r"Violet Point: $(i,j+1)$"),
    Line2D([0], [0], marker='o', linestyle='None', markerfacecolor='green', markeredgecolor='green', markersize=8, label=r"Green Point: $(i-1,j)$"),
    Line2D([0], [0], marker='o', linestyle='None', markerfacecolor='purple', markeredgecolor='purple', markersize=8, label=r"Purple Point: $(i+1,j)$"),
]

      
ax.legend(handles=legend_handles, loc="lower center", bbox_to_anchor=(0.5, -0.32), ncol=3, frameon=True, fontsize=10, title="Stencil Point Index")

      
# ------------------------------------------------------------
# Formatting
# ------------------------------------------------------------
ax.set_aspect("equal", adjustable="box")
ax.set_xlim(xmin - 0.03, xmax + 0.03)
ax.set_ylim(ymin_ref - 0.03, ymax_ref + 0.03)

      
ax.set_xlabel(r"$x$")
ax.set_ylabel(r"$y$")
ax.set_title(r"Rectangular Domain with 5-Point Stencil with uniform $\Delta x$, $\Delta y$, with Symmetry about Y axis")

      
plt.show()

# Task (A)

For the reduced half-domain,
$$
0 \le x \le \frac{a}{2},
\qquad
0 \le y \le b,
$$
with a uniform grid spacing
$$
\Delta x = \Delta y = h,
$$
the number of grid points in the $x$-direction is
$$
\boxed{
n_x = \frac{a/2}{h} + 1
= \frac{a}{2h}+1
}
$$
and the number of grid points in the $y$-direction is
$$
\boxed{
n_y = \frac{b}{h}+1
}
$$
where $n_x$ and $n_y$ include the boundary points. Therefore,
$$
\boxed{
n_x = \frac{a}{2h}+1,
\qquad
n_y = \frac{b}{h}+1
}
$$
given that
$$
\frac{a}{2h}
\qquad \text{and} \qquad
\frac{b}{h}
$$
are integers.

#### Matrix is given as:

Define
$$
N_x=n_x-1,
$$
This is because the last column of the rectangular grid will have no finite difference form as it is a dirichlet boundary condition (i=nx)
$$
N_y=n_y-2,
$$
This is because the top and bottom row of the rectangular grid will have no finite difference form as it is a dirichlet boundary condition (j = 0 and j = ny)

so that the total number of unknown temperatures is

$$
N=N_xN_y=(n_x-1)(n_y-2).
$$

The unknown vector is ordered row-by-row as

# $$
\mathbf{x}

\begin{bmatrix}
T_{0,1} \
T_{1,1} \
T_{2,1} \
\vdots \
T_{N_x-1,1} \
T_{0,2} \
T_{1,2} \
\vdots \
T_{N_x-1,N_y}
\end{bmatrix}.
$$

For a uniform square mesh, $\Delta x=\Delta y=h$, the system is

$$
\boxed{\mathbf{A}\mathbf{x}=\mathbf{b}}
$$

where

# Discretized Linear System

For a 9 rows × 10 columns grid i.e. h = 0.02, the linear system $\mathbf{A}\mathbf{x}=\mathbf{b}$ is:

Let (\mathbf{D}) be the (10\times 10) horizontal‑direction matrix (left boundary: Neumann, right boundary: Dirichlet):

$$
\mathbf{D}=
\begin{bmatrix}
-4 & 2 & 0 & 0 & \cdots & 0 & 0 & 0 \
1 & -4 & 1 & 0 & \cdots & 0 & 0 & 0 \
0 & 1 & -4 & 1 & \cdots & 0 & 0 & 0 \
\vdots & \vdots & \vdots & \vdots & \ddots & \vdots & \vdots & \vdots \
0 & 0 & 0 & 0 & \cdots & 1 & -4 & 1 \
0 & 0 & 0 & 0 & \cdots & 0 & 1 & -4
\end{bmatrix}_{10\times 10}
$$

and (\mathbf{I}) the (10\times 10) identity matrix.

Then the (90\times 90) matrix (\mathbf{A}) is block‑tridiagonal with 9 block rows and columns:

$$
\mathbf{A}=
\begin{bmatrix}
\mathbf{D} & \mathbf{I} & \mathbf{0} & \cdots & \mathbf{0} \
\mathbf{I} & \mathbf{D} & \mathbf{I} & \ddots & \vdots \
\mathbf{0} & \mathbf{I} & \mathbf{D} & \ddots & \mathbf{0} \
\vdots & \ddots & \ddots & \ddots & \mathbf{I} \
\mathbf{0} & \cdots & \mathbf{0} & \mathbf{I} & \mathbf{D}
\end{bmatrix}.
$$

Unknown vector (row‑wise ordering, (i)=row, (j)=column):

$$
\mathbf{x}=
\begin{bmatrix}
T_{0,0}, T_{0,1}, \ldots, T_{0,9}, ;
T_{1,0}, \ldots, T_{1,9}, ;
\ldots, ;
T_{8,0}, \ldots, T_{8,9}
\end{bmatrix}^T.
$$

Right‑hand side (source term on the bottom row only):

$$
\mathbf{b}=
\begin{bmatrix}
\underbrace{0,\ldots,0}{80\text{ entries}},
\underbrace{-100,\ldots,-100}{10\text{ entries}}
\end{bmatrix}^T.
$$

Thus the system is fully defined without expanding the (90\times 90) matrix explicitly.

#### Prescribed Boundary Conditions

Bottom boundary:

$$
T_{i,0}=0,
\qquad
i=0,1,\ldots,n_x-1
$$

Right boundary:

$$
T_{n_x-1,j}=0,
\qquad
j=0,1,\ldots,n_y-1
$$

Top boundary:

$$
T_{i,n_y-1}=T_0=100^\circ\mathrm{C},
\qquad
i=0,1,\ldots,n_x-1
$$

The left boundary is the symmetry boundary and is therefore not Dirichlet:

$$
\left.
\frac{\partial T}{\partial x}
\right|_{i=0}
=0.
$$

with

$$
T_0=100^\circ\mathrm C.
$$

Thus,

# $$
\boxed{
\mathbf A_{,N\times N}
\mathbf x_{,N\times1}

\mathbf b_{,N\times1},
\qquad
N=(n_x-1)(n_y-2)
}
$$

### Variable and Mesh Setup

Following section allows to edit with the mesh grid and boundary conditions to solve for various cases for a similar domain.

a = 0.4/2 # Length of the rectangular domain
b = 0.2 # Width of the rectangular domain

      
TNorthWall = 100 # Temperature at the North wall
TSouthWall = 0 # Temperature at the South wall
TEastWall = 0 # Temperature at the East wall
TWestWall = 0 # Temperature at the West wall // Not used as we are exploiting symmetry.

      

      
N = 0 # Total number of unknowns in the domain (excluding boundary points)
nX = 0 # Number of grid points in x direction
nY = 0 # Number of grid points in y direction
linaearizedMeshGrid =  {}
meshGrid = []
unifiedIndexMap = []

      
def MeshSetup(a, b, h):

      
    global nX
    global nY
    nX = int(a/h) + 1 # Number of grid points in x direction
    global nY
    nY = int(b/h) + 1 # Number of grid points in y direction

      
    i = range(0, nX-1) # x index for interior points
    j = range(1, nY-2) # y index for interior points

      
    global N
    N = (nX-1)*(nY-2) # Total number of unknowns in the domain (excluding boundary points)

      
    global linaearizedMeshGrid
    global meshGrid
    global unifiedIndexMap

      
    # Defined a meshGrid dictionary to store the mapping of grid point indices to their corresponding (row, column) coordinates in the mesh.
    # The loop iterates over the interior points of the grid (excluding boundary points) and populates the meshGrid dictionary with the appropriate mappings.
    linaearizedMeshGrid =  {}
    meshGrid = np.zeros((nY-2,nX-1), dtype=float)
    unifiedIndexMap = []

      
    for rowId in range(nY - 2):
        for columnId in range(0, nX - 1):
            unifiedIndex = columnId + (nX - 1)*rowId
            unifiedIndexMap.append(unifiedIndex)
            if rowId == 0:
                linaearizedMeshGrid[unifiedIndex] = [meshGrid[rowId][columnId], "SouthEnd"]
            elif rowId == nY - 3:
                linaearizedMeshGrid[unifiedIndex] = [meshGrid[rowId][columnId], "NorthEnd"]
            elif columnId == 0:
                linaearizedMeshGrid[unifiedIndex] = [meshGrid[rowId][columnId], "WestEnd"]
            elif columnId == nX - 2:
                linaearizedMeshGrid[unifiedIndex] = [meshGrid[rowId][columnId], "EastEnd"]
            else:
                linaearizedMeshGrid[unifiedIndex] = [meshGrid[rowId][columnId], "Interior"]

      
            if unifiedIndex == 0:
                linaearizedMeshGrid[unifiedIndex][1] = "SymmetricSouthWestCorner"
            if unifiedIndex == nX - 2:
                linaearizedMeshGrid[unifiedIndex][1] = "DirichletSouthEastCorner"
            if unifiedIndex == (nX - 1)*(nY - 2) - (nX - 1):
                linaearizedMeshGrid[unifiedIndex][1] = "SymmetricNorthWestCorner"
            if unifiedIndex == (nX - 1)*(nY - 2) - 1:
                linaearizedMeshGrid[unifiedIndex][1] = "DirichletNorthEastCorner"

### Mesh Validation

def MeshValidation(a,b,h):
    # Physical coordinates
    x_coords = np.linspace(0, a-h, nX-1)   # columns: 0 to nX-2
    y_coords = np.linspace(h, b-h, nY-2)   # rows:    1 to nY-2 in physical space

      
    # ------------------------------------------------------------
    # Colors for each node type
    # ------------------------------------------------------------
    #Set colors for mesh node type:
    colors = {
        "SouthEnd": "red",
        "NorthEnd": "blue",
        "WestEnd": "green",
        "EastEnd": "orange",
        "Interior": "gray",
        "SymmetricSouthWestCorner": "purple",
        "DirichletSouthEastCorner": "cyan",
        "SymmetricNorthWestCorner": "magenta",
        "DirichletNorthEastCorner": "yellow"
    }
    # ------------------------------------------------------------
    # Plot the grid points
    # ------------------------------------------------------------
    plt.figure(figsize=(16, 8))

      
    for rowId in range(nY - 2):
        for columnId in range(0, nX - 1):
            unifiedIndex = columnId + (nX - 1)*rowId
            nodeType = linaearizedMeshGrid[unifiedIndex][1]
            x = x_coords[columnId]
            y = y_coords[rowId]
            plt.scatter(x, y, color=colors[nodeType], s=80)
            plt.text(x + 0.002, y + 0.002, str(unifiedIndex), fontsize=8)

      
    # Domain outline
    plt.plot([0, a, a, 0, 0], [0, 0, b, b, 0], 'k--', linewidth=1)

      
    # Legend
    for label, color in colors.items():
        plt.scatter([], [], color=color, s=80, label=label)

      
    plt.xlabel("x")
    plt.ylabel("y")
    plt.title("Grid Points with Boundary Classification and Unified Index")
    plt.axis("equal")
    plt.grid(True)

      
    # Put legend below the plot
    plt.legend(
        loc="upper center",
        bbox_to_anchor=(0.5, -0.15),
        ncol=3,
        fontsize=10,
        frameon=True
    )

      
    # Add space at bottom for legend
    plt.tight_layout(rect=[0, 0.08, 1, 1])

      
    plt.show()

### Setting Up System Matrix

Following section now uses the carefully setup linearMesgGrid, stored as a dictionary, for ease of naming and indexing and generates the Matrix Equations which will give us the solution to our problem at hand.

#Data Structures for Finite Difference Stencil
class StencilPointIds:
    def __init__(self, point, N, S, E, W):
        self.point = point
        self.N = N
        self.S = S
        self.E = E
        self.W = W

      
def ComputeNSEWPointIds(meshPointId):
    # Compute the North, South, East, and West point IDs based on the current meshPointId
    northId = meshPointId + (nX - 1) if (meshPointId + (nX - 1)) < N else None
    southId = meshPointId - (nX - 1) if (meshPointId - (nX - 1)) >= 0 else None
    eastId = meshPointId + 1 if (meshPointId + 1) % (nX - 1) != 0 else None
    westId = meshPointId - 1 if meshPointId % (nX - 1) != 0 else None

      
    return StencilPointIds(meshPointId, northId, southId, eastId, westId)

      
def GenerateSymmetricBoundaryCornerEquations(meshPointId):
    dependencyMap = ComputeNSEWPointIds(meshPointId)
    if(linaearizedMeshGrid[meshPointId][1] == "SymmetricSouthWestCorner"):
        systemMatrix[meshPointId, dependencyMap.point] = -4.0;
        systemMatrix[meshPointId, dependencyMap.N] = 1.0;
        systemMatrix[meshPointId, dependencyMap.E] = 2.0;
        temperatureVector[dependencyMap.point] = -TSouthWall;
    elif linaearizedMeshGrid[meshPointId][1] == "SymmetricNorthWestCorner":
        systemMatrix[meshPointId,dependencyMap.point] = -4.0;
        systemMatrix[meshPointId,dependencyMap.S] = 1.0;
        systemMatrix[meshPointId,dependencyMap.E] = 2.0;
        temperatureVector[dependencyMap.point] = -TNorthWall;
    elif linaearizedMeshGrid[meshPointId][1] == "WestEnd":
        systemMatrix[meshPointId,dependencyMap.point] = -4.0;
        systemMatrix[meshPointId,dependencyMap.N] = 1.0;
        systemMatrix[meshPointId,dependencyMap.S] = 1.0;
        systemMatrix[meshPointId,dependencyMap.E] = 2.0;
        temperatureVector[dependencyMap.point] = 0.0;

      
def GenerateDirichletBoundaryCornerEquations(meshPointId):
    dependencyMap = ComputeNSEWPointIds(meshPointId)
    if (linaearizedMeshGrid[meshPointId][1] == "NorthEnd"):
        systemMatrix[meshPointId, dependencyMap.point] = -4.0;
        systemMatrix[meshPointId, dependencyMap.S] = 1.0;
        systemMatrix[meshPointId, dependencyMap.W] = 1.0;
        systemMatrix[meshPointId, dependencyMap.E] = 1.0;
        temperatureVector[dependencyMap.point] = -TNorthWall;
    elif (linaearizedMeshGrid[meshPointId][1] == "SouthEnd"):
        systemMatrix[meshPointId, dependencyMap.point] = -4.0;
        systemMatrix[meshPointId, dependencyMap.N] = 1.0;
        systemMatrix[meshPointId, dependencyMap.W] = 1.0;
        systemMatrix[meshPointId, dependencyMap.E] = 1.0;
        temperatureVector[dependencyMap.point] = -TSouthWall;
    elif (linaearizedMeshGrid[meshPointId][1] == "EastEnd"):
        systemMatrix[meshPointId, dependencyMap.point] = -4.0;
        systemMatrix[meshPointId, dependencyMap.N] = 1.0;
        systemMatrix[meshPointId, dependencyMap.S] = 1.0;
        systemMatrix[meshPointId, dependencyMap.W] = 1.0;
        temperatureVector[dependencyMap.point] = -TEastWall;
    elif (linaearizedMeshGrid[meshPointId][1] == "DirichletSouthEastCorner"):
        systemMatrix[meshPointId, dependencyMap.point] = -4.0;
        systemMatrix[meshPointId, dependencyMap.N] = 1.0;
        systemMatrix[meshPointId, dependencyMap.W] = 1.0;
        temperatureVector[dependencyMap.point] = -TEastWall - TSouthWall;
    elif (linaearizedMeshGrid[meshPointId][1] == "DirichletNorthEastCorner"):
        systemMatrix[meshPointId, dependencyMap.point] = -4.0;
        systemMatrix[meshPointId, dependencyMap.S] = 1.0;
        systemMatrix[meshPointId, dependencyMap.W] = 1.0;
        temperatureVector[dependencyMap.point] = -TEastWall - TNorthWall;

      
def GenerateInternalNodeEquations(meshPointId):
    dependencyMap = ComputeNSEWPointIds(meshPointId)
    if (linaearizedMeshGrid[meshPointId][1] == "Interior"):
        systemMatrix[meshPointId, dependencyMap.point] = -4.0;
        systemMatrix[meshPointId, dependencyMap.N] = 1.0;
        systemMatrix[meshPointId, dependencyMap.S] = 1.0;
        systemMatrix[meshPointId, dependencyMap.E] = 1.0;
        systemMatrix[meshPointId, dependencyMap.W] = 1.0;
        temperatureVector[dependencyMap.point] = 0.0;

      
def GenerateSystemOfEquations():
    for(meshPointId) in range(N):
        if linaearizedMeshGrid[meshPointId][1] == "SymmetricSouthWestCorner" or linaearizedMeshGrid[meshPointId][1] == "SymmetricNorthWestCorner" or linaearizedMeshGrid[meshPointId][1] == "WestEnd":
            GenerateSymmetricBoundaryCornerEquations(meshPointId)
        elif linaearizedMeshGrid[meshPointId][1] == "NorthEnd" or linaearizedMeshGrid[meshPointId][1] == "SouthEnd" or linaearizedMeshGrid[meshPointId][1] == "EastEnd" or linaearizedMeshGrid[meshPointId][1] == "DirichletSouthEastCorner" or linaearizedMeshGrid[meshPointId][1] == "DirichletNorthEastCorner":
            GenerateDirichletBoundaryCornerEquations(meshPointId)
        elif linaearizedMeshGrid[meshPointId][1] == "Interior":
            GenerateInternalNodeEquations(meshPointId)
    print("System Matrix is Generated. The shape of the system matrix is:", systemMatrix.shape)
    print("Temperature Vector is Generated. The shape of the temperature vector is:", temperatureVector.shape)

      
print("Funtion Setup for system of equations is complete. Please call the functions in the following order:")

Funtion Setup for system of equations is complete. Please call the functions in the following order:

# Task B

$$
h = 0.02,; 0.01,; 0.005\ \mathrm{m}.
$$

### Solve the System Using LU Decomposition

import Logger
import importlib

      
importlib.reload(Logger)

      
meshStepSizes = [
    0.02,
    0.01,
    0.005
]

      
meshRefinementResults = Logger.RunMeshRefinementStudy(
    a=a,
    b=b,
    meshStepSizes=meshStepSizes,
    MeshSetup=MeshSetup,
    MeshValidation=MeshValidation,
    GenerateSystemOfEquations=GenerateSystemOfEquations,
    notebookGlobals=globals(),
    RunMeshValidation=True
)

      
from contextlib import redirect_stdout

      
with open("../../Dumps/Problem1/MeshRefinementResults.txt", "w") as outputFile, redirect_stdout(outputFile):
    meshRefinementResults = Logger.RunMeshRefinementStudy(
        a=a,
        b=b,
        meshStepSizes=meshStepSizes,
        MeshSetup=MeshSetup,
        MeshValidation=MeshValidation,
        GenerateSystemOfEquations=GenerateSystemOfEquations,
        notebookGlobals=globals(),
        RunMeshValidation=False
    )

====================================================================================================
                                   STARTING MESH REFINEMENT STUDY                                   
====================================================================================================
Number of mesh refinements                                 : 3
Mesh step sizes                                            : 0.02000 m, 0.01000 m, 0.00500 m

System Matrix is Generated. The shape of the system matrix is: (90, 90)
Temperature Vector is Generated. The shape of the temperature vector is: (90, 1)

      
====================================================================================================
                       FINITE DIFFERENCE PERFORMANCE REPORT — h = 0.020000 m                        
====================================================================================================

      
----------------------------------------------------------------------------------------------------
MESH INFORMATION
----------------------------------------------------------------------------------------------------
Domain length in x-direction                               : 0.200000 m
Domain length in y-direction                               : 0.200000 m
Computational domain area                                  : 0.040000 m^2

      
Requested mesh spacing, h                                  : 0.020000 m
Actual x-direction spacing, dx                             : 0.020000 m
Actual y-direction spacing, dy                             : 0.020000 m

      
Grid points in x-direction, nX                             : 11
Grid points in y-direction, nY                             : 11
x-direction intervals                                      : 10
y-direction intervals                                      : 10
Computational columns                                      : 10
Computational rows                                         : 9
Total unknowns, N                                          : 90

      
----------------------------------------------------------------------------------------------------
COMPUTATIONAL NODE CLASSIFICATION
----------------------------------------------------------------------------------------------------
Interior nodes                                             : 56
North boundary nodes                                       : 8
South boundary nodes                                       : 8
East boundary nodes                                        : 7
West symmetry boundary nodes                               : 7
Symmetric south-west corner                                : 1
Symmetric north-west corner                                : 1
Dirichlet south-east corner                                : 1
Dirichlet north-east corner                                : 1

      
Total classified nodes                                     : 90
Expected number of nodes, N                                : 90
Node classification consistency                            : PASS

      
----------------------------------------------------------------------------------------------------
LINEAR SYSTEM INFORMATION
----------------------------------------------------------------------------------------------------
System matrix dimensions                                   : 90 x 90
Temperature vector dimensions                              : 90 x 1
Number of equations                                        : 90
Matrix data type                                           : float64

      
Total matrix coefficients                                  : 8,100
Non-zero matrix coefficients                               : 412
Matrix sparsity                                            : 94.9136 %

      
Exact system matrix storage                                : 0.061798 MB
Exact temperature vector storage                           : 0.000687 MB
Exact matrix + RHS storage                                 : 0.062485 MB

      
----------------------------------------------------------------------------------------------------
PERFORMANCE OF MAJOR COMPUTATIONAL STEPS
----------------------------------------------------------------------------------------------------
Computational Step                        CPU Time [s]   Wall Time [s] Delta Memory [MB]
----------------------------------------------------------------------------------------
1. Mesh Setup                                 0.000044        0.000044             0.000
2. Mesh Validation                            0.251614        0.251873             9.039
3. Matrix / RHS Allocation                    0.000018        0.000018             0.000
4. System Matrix Setup                        0.000149        0.000137             0.000
5. LU Factorization                           0.000185        0.000182             0.426
6. System Solution                            0.000062        0.000062             0.188
----------------------------------------------------------------------------------------
TOTAL                                         0.252523        0.252687             9.656

      
----------------------------------------------------------------------------------------------------
DIRECT SOLVER SUMMARY
----------------------------------------------------------------------------------------------------
Solution method                                            : LU decomposition with partial pivoting

      
LU factorization CPU time                                  : 0.000185 s
System solution CPU time                                   : 0.000062 s
Total direct solver CPU time                               : 0.000247 s

      
LU factorization wall time                                 : 0.000182 s
System solution wall time                                  : 0.000062 s
Total direct solver wall time                              : 0.000244 s

      
----------------------------------------------------------------------------------------------------
MEMORY USAGE
----------------------------------------------------------------------------------------------------
Process memory before simulation                           : 123.332 MB
Process memory after simulation                            : 132.988 MB
Net process memory change                                  : 9.656 MB

      
System matrix storage                                      : 0.061798 MB
Temperature vector storage                                 : 0.000687 MB
Factorized system matrix storage                           : 0.061798 MB
Pivot indices storage                                      : 0.000343 MB
Temperature solution vector storage                        : 0.000687 MB
Total tracked numerical storage                            : 0.125313 MB

      
----------------------------------------------------------------------------------------------------
NUMERICAL VERIFICATION
----------------------------------------------------------------------------------------------------
Temperature solution vector shape                          : (90, 1)
Temperature index range                                    : 0 to 89
Minimum temperature                                        : 1.55917038
Maximum temperature                                        : 88.18601578
Residual norm ||A*T - b||                                  : 2.171178e-13

      
----------------------------------------------------------------------------------------------------
TEMPERATURE DATA OUTPUT
----------------------------------------------------------------------------------------------------
Temperature CSV                                            : ../../Dumps/Problem1/TemperatureSolution_h_0.02000.csv
CSV index range                                            : 0 to 89
CSV number of temperature values                           : 90

      
====================================================================================================
                                        SIMULATION COMPLETE                                         
====================================================================================================

System Matrix is Generated. The shape of the system matrix is: (380, 380)
Temperature Vector is Generated. The shape of the temperature vector is: (380, 1)

      
====================================================================================================
                       FINITE DIFFERENCE PERFORMANCE REPORT — h = 0.010000 m                        
====================================================================================================

      
----------------------------------------------------------------------------------------------------
MESH INFORMATION
----------------------------------------------------------------------------------------------------
Domain length in x-direction                               : 0.200000 m
Domain length in y-direction                               : 0.200000 m
Computational domain area                                  : 0.040000 m^2

      
Requested mesh spacing, h                                  : 0.010000 m
Actual x-direction spacing, dx                             : 0.010000 m
Actual y-direction spacing, dy                             : 0.010000 m

      
Grid points in x-direction, nX                             : 21
Grid points in y-direction, nY                             : 21
x-direction intervals                                      : 20
y-direction intervals                                      : 20
Computational columns                                      : 20
Computational rows                                         : 19
Total unknowns, N                                          : 380

      
----------------------------------------------------------------------------------------------------
COMPUTATIONAL NODE CLASSIFICATION
----------------------------------------------------------------------------------------------------
Interior nodes                                             : 306
North boundary nodes                                       : 18
South boundary nodes                                       : 18
East boundary nodes                                        : 17
West symmetry boundary nodes                               : 17
Symmetric south-west corner                                : 1
Symmetric north-west corner                                : 1
Dirichlet south-east corner                                : 1
Dirichlet north-east corner                                : 1

      
Total classified nodes                                     : 380
Expected number of nodes, N                                : 380
Node classification consistency                            : PASS

      
----------------------------------------------------------------------------------------------------
LINEAR SYSTEM INFORMATION
----------------------------------------------------------------------------------------------------
System matrix dimensions                                   : 380 x 380
Temperature vector dimensions                              : 380 x 1
Number of equations                                        : 380
Matrix data type                                           : float64

      
Total matrix coefficients                                  : 144,400
Non-zero matrix coefficients                               : 1,822
Matrix sparsity                                            : 98.7382 %

      
Exact system matrix storage                                : 1.101685 MB
Exact temperature vector storage                           : 0.002899 MB
Exact matrix + RHS storage                                 : 1.104584 MB

      
----------------------------------------------------------------------------------------------------
PERFORMANCE OF MAJOR COMPUTATIONAL STEPS
----------------------------------------------------------------------------------------------------
Computational Step                        CPU Time [s]   Wall Time [s] Delta Memory [MB]
----------------------------------------------------------------------------------------
1. Mesh Setup                                 0.000134        0.000134             0.012
2. Mesh Validation                            0.693111        0.695032            20.316
3. Matrix / RHS Allocation                    0.000112        0.000087             0.000
4. System Matrix Setup                        0.000469        0.000461             0.000
5. LU Factorization                           0.013539        0.002095             1.922
6. System Solution                            0.000138        0.000079             0.000
----------------------------------------------------------------------------------------
TOTAL                                         0.732198        0.698194            22.254

      
----------------------------------------------------------------------------------------------------
DIRECT SOLVER SUMMARY
----------------------------------------------------------------------------------------------------
Solution method                                            : LU decomposition with partial pivoting

      
LU factorization CPU time                                  : 0.013539 s
System solution CPU time                                   : 0.000138 s
Total direct solver CPU time                               : 0.013676 s

      
LU factorization wall time                                 : 0.002095 s
System solution wall time                                  : 0.000079 s
Total direct solver wall time                              : 0.002174 s

      
----------------------------------------------------------------------------------------------------
MEMORY USAGE
----------------------------------------------------------------------------------------------------
Process memory before simulation                           : 132.992 MB
Process memory after simulation                            : 155.246 MB
Net process memory change                                  : 22.254 MB

      
System matrix storage                                      : 1.101685 MB
Temperature vector storage                                 : 0.002899 MB
Factorized system matrix storage                           : 1.101685 MB
Pivot indices storage                                      : 0.001450 MB
Temperature solution vector storage                        : 0.002899 MB
Total tracked numerical storage                            : 2.210617 MB

      
----------------------------------------------------------------------------------------------------
NUMERICAL VERIFICATION
----------------------------------------------------------------------------------------------------
Temperature solution vector shape                          : (380, 1)
Temperature index range                                    : 0 to 379
Minimum temperature                                        : 0.38761747
Maximum temperature                                        : 94.09687399
Residual norm ||A*T - b||                                  : 5.633659e-13

      
----------------------------------------------------------------------------------------------------
TEMPERATURE DATA OUTPUT
----------------------------------------------------------------------------------------------------
Temperature CSV                                            : ../../Dumps/Problem1/TemperatureSolution_h_0.01000.csv
CSV index range                                            : 0 to 379
CSV number of temperature values                           : 380

      
====================================================================================================
                                        SIMULATION COMPLETE                                         
====================================================================================================

System Matrix is Generated. The shape of the system matrix is: (1560, 1560)
Temperature Vector is Generated. The shape of the temperature vector is: (1560, 1)

      
====================================================================================================
                       FINITE DIFFERENCE PERFORMANCE REPORT — h = 0.005000 m                        
====================================================================================================

      
----------------------------------------------------------------------------------------------------
MESH INFORMATION
----------------------------------------------------------------------------------------------------
Domain length in x-direction                               : 0.200000 m
Domain length in y-direction                               : 0.200000 m
Computational domain area                                  : 0.040000 m^2

      
Requested mesh spacing, h                                  : 0.005000 m
Actual x-direction spacing, dx                             : 0.005000 m
Actual y-direction spacing, dy                             : 0.005000 m

      
Grid points in x-direction, nX                             : 41
Grid points in y-direction, nY                             : 41
x-direction intervals                                      : 40
y-direction intervals                                      : 40
Computational columns                                      : 40
Computational rows                                         : 39
Total unknowns, N                                          : 1,560

      
----------------------------------------------------------------------------------------------------
COMPUTATIONAL NODE CLASSIFICATION
----------------------------------------------------------------------------------------------------
Interior nodes                                             : 1,406
North boundary nodes                                       : 38
South boundary nodes                                       : 38
East boundary nodes                                        : 37
West symmetry boundary nodes                               : 37
Symmetric south-west corner                                : 1
Symmetric north-west corner                                : 1
Dirichlet south-east corner                                : 1
Dirichlet north-east corner                                : 1

      
Total classified nodes                                     : 1,560
Expected number of nodes, N                                : 1,560
Node classification consistency                            : PASS

      
----------------------------------------------------------------------------------------------------
LINEAR SYSTEM INFORMATION
----------------------------------------------------------------------------------------------------
System matrix dimensions                                   : 1,560 x 1,560
Temperature vector dimensions                              : 1,560 x 1
Number of equations                                        : 1,560
Matrix data type                                           : float64

      
Total matrix coefficients                                  : 2,433,600
Non-zero matrix coefficients                               : 7,642
Matrix sparsity                                            : 99.6860 %

      
Exact system matrix storage                                : 18.566895 MB
Exact temperature vector storage                           : 0.011902 MB
Exact matrix + RHS storage                                 : 18.578796 MB

      
----------------------------------------------------------------------------------------------------
PERFORMANCE OF MAJOR COMPUTATIONAL STEPS
----------------------------------------------------------------------------------------------------
Computational Step                        CPU Time [s]   Wall Time [s] Delta Memory [MB]
----------------------------------------------------------------------------------------
1. Mesh Setup                                 0.001811        0.000526             0.000
2. Mesh Validation                            3.729411        2.860269            58.766
3. Matrix / RHS Allocation                    0.000030        0.000030             0.004
4. System Matrix Setup                        0.003336        0.003241            16.961
5. LU Factorization                           0.974353        0.050144            23.719
6. System Solution                            0.022972        0.000986             0.000
----------------------------------------------------------------------------------------
TOTAL                                         5.063236        2.928674            99.824

      
----------------------------------------------------------------------------------------------------
DIRECT SOLVER SUMMARY
----------------------------------------------------------------------------------------------------
Solution method                                            : LU decomposition with partial pivoting

      
LU factorization CPU time                                  : 0.974353 s
System solution CPU time                                   : 0.022972 s
Total direct solver CPU time                               : 0.997326 s

      
LU factorization wall time                                 : 0.050144 s
System solution wall time                                  : 0.000986 s
Total direct solver wall time                              : 0.051129 s

      
----------------------------------------------------------------------------------------------------
MEMORY USAGE
----------------------------------------------------------------------------------------------------
Process memory before simulation                           : 152.188 MB
Process memory after simulation                            : 252.012 MB
Net process memory change                                  : 99.824 MB

      
System matrix storage                                      : 18.566895 MB
Temperature vector storage                                 : 0.011902 MB
Factorized system matrix storage                           : 18.566895 MB
Pivot indices storage                                      : 0.005951 MB
Temperature solution vector storage                        : 0.011902 MB
Total tracked numerical storage                            : 37.163544 MB

      
----------------------------------------------------------------------------------------------------
NUMERICAL VERIFICATION
----------------------------------------------------------------------------------------------------
Temperature solution vector shape                          : (1560, 1)
Temperature index range                                    : 0 to 1559
Minimum temperature                                        : 0.09676357
Maximum temperature                                        : 97.04896763
Residual norm ||A*T - b||                                  : 1.336582e-12

      
----------------------------------------------------------------------------------------------------
TEMPERATURE DATA OUTPUT
----------------------------------------------------------------------------------------------------
Temperature CSV                                            : ../../Dumps/Problem1/TemperatureSolution_h_0.00500.csv
CSV index range                                            : 0 to 1559
CSV number of temperature values                           : 1,560

      
====================================================================================================
                                        SIMULATION COMPLETE                                         
====================================================================================================

      
====================================================================================================
                               MESH REFINEMENT PERFORMANCE COMPARISON                               
====================================================================================================
     h [m]           N     Matrix MB     Setup CPU        LU CPU     Solve CPU    Direct CPU     Total CPU
----------------------------------------------------------------------------------------------------------
   0.02000          90         0.062      0.000149      0.000185      0.000062      0.000247      0.252523
   0.01000         380         1.102      0.000469      0.013539      0.000138      0.013676      0.732198
   0.00500       1,560        18.567      0.003336      0.974353      0.022972      0.997326      5.063236
----------------------------------------------------------------------------------------------------------

      
----------------------------------------------------------------------------------------------------
OUTPUT FILES
----------------------------------------------------------------------------------------------------
Performance report                                         : ../../Dumps/Problem1/MeshRefinementResults.txt
Temperature CSV for h = 0.02000 m                          : ../../Dumps/Problem1/TemperatureSolution_h_0.02000.csv
Temperature CSV for h = 0.01000 m                          : ../../Dumps/Problem1/TemperatureSolution_h_0.01000.csv
Temperature CSV for h = 0.00500 m                          : ../../Dumps/Problem1/TemperatureSolution_h_0.00500.csv

      
====================================================================================================
                                   MESH REFINEMENT STUDY COMPLETE                                   
====================================================================================================

length, width, halfLength = 0.4, 0.2, 0.2
TNorthWall, TSouthWall, TEastWall, TWestWall = 100.0, 0.0, 0.0, 0.0
hValues = [0.02, 0.01, 0.005]
dumpPath = Path("../../Dumps/Problem1")

      
fig = plt.figure(figsize=(19, 6))
axes = []

      
for plotId, h in enumerate(hValues, 1):
    temperatureValues = np.loadtxt(dumpPath / f"TemperatureSolution_h_{h:.5f}.csv", delimiter=",", skiprows=1, usecols=1)

      
    xHalf = np.arange(0, halfLength, h); yInterior = np.arange(h, width, h)
    THalf = temperatureValues.reshape(len(yInterior), len(xHalf))

      
    # Mirror about physical symmetry plane x = 0.2 m
    xRight = halfLength + xHalf
    xInterior = np.concatenate((length - xRight[:0:-1], xRight))
    TInterior = np.concatenate((THalf[:, :0:-1], THalf), axis=1)

      
    # Add physical boundaries x = 0, x = 0.4, y = 0 and y = 0.2
    xFull = np.concatenate(([0.0], xInterior, [length]))
    yFull = np.concatenate(([0.0], yInterior, [width]))
    TFull = np.zeros((len(yFull), len(xFull)))

      
    TFull[1:-1, 1:-1] = TInterior
    TFull[1:-1, 0] = TWestWall
    TFull[1:-1, -1] = TEastWall
    TFull[0, :] = TSouthWall
    TFull[-1, :] = TNorthWall

      
    XFull, YFull = np.meshgrid(xFull, yFull)

      
    axis = fig.add_subplot(1, 3, plotId, projection="3d"); axes.append(axis)
    axis.plot_surface(XFull, YFull, TFull, cmap="coolwarm", vmin=0, vmax=100, alpha=0.72, edgecolor="none")
    axis.scatter(XFull, YFull, TFull, c=TFull, cmap="coolwarm", vmin=0, vmax=100, s=10)

      
    axis.set_xlabel("x [m]"); axis.set_ylabel("y [m]"); axis.set_zlabel("Temperature [°C]")
    axis.set_title(f"h = {h:.3f} m")
    axis.set_xlim(0, length); axis.set_ylim(0, width); axis.set_zlim(0, 100)

      
fig.colorbar(ScalarMappable(norm=Normalize(0, 100), cmap="coolwarm"), ax=axes, shrink=0.70, pad=0.06, label="Temperature [°C]")
fig.suptitle("Temperature Distribution: Symmetry-Reconstructed and Shifted back to original Domain", fontsize=15)
plt.show()

## Task C

#### Comparison with Analytical Solution

import numpy as np

      
def AnalyticalSolution(x, y, a, b, T0, numberOfTerms):
    temperature = 0.0
    for termId in range(numberOfTerms):
        n = 2*termId + 1
        # This form avoids overflow for large values of n
        sinhRatio = np.exp((n*np.pi/a)*(y-b))*(1-np.exp(-2*n*np.pi*y/a))/(1-np.exp(-2*n*np.pi*b/a))
        temperature += (1/n)*np.sin(n*np.pi*x/a)*sinhRatio
    return (4*T0/np.pi)*temperature

      

      
def FindNumberOfTermsForConvergence(a, b, T0, h=0.005, tolerance=1e-5):
    # Use the smallest mesh because it has the largest number of comparison points
    xCoordinates = np.arange(h, a, h)
    yCoordinates = np.arange(h, b, h)
    numberOfTerms = 10
    while True:
        maximumAbsoluteError = 0.0
        maximumRelativeError = 0.0
        worstPoint = None
        # Check convergence at every interior point on the finest mesh
        for y in yCoordinates:
            for x in xCoordinates:
                previousTemperature = AnalyticalSolution(x, y, a, b, T0, numberOfTerms)
                currentTemperature = AnalyticalSolution(x, y, a, b, T0, numberOfTerms + 10)
                absoluteError = abs(currentTemperature-previousTemperature)
                relativeError = absoluteError/abs(currentTemperature) if abs(currentTemperature) > 1e-12 else 0.0
                # Store the largest error found on the mesh
                if absoluteError > maximumAbsoluteError:
                    maximumAbsoluteError = absoluteError
                    worstPoint = (x, y)
                if relativeError > maximumRelativeError:
                    maximumRelativeError = relativeError
        print(
            f"Terms: {numberOfTerms:3d} -> {numberOfTerms+10:3d} | "
            f"Max. Absolute Error: {maximumAbsoluteError:.8e} | "
            f"Max. Relative Error: {maximumRelativeError:.8e} | "
            f"Worst Point: ({worstPoint[0]:.3f}, {worstPoint[1]:.3f})"
        )
        # The series is considered converged when every point satisfies the tolerance
        if maximumAbsoluteError < tolerance:
            numberOfTerms += 10
            break

      
        numberOfTerms += 10

      
    print(f"\nAnalytical solution converged using {numberOfTerms} odd terms.")
    print(f"Highest value of n used = {2*numberOfTerms-1}")
    print(f"Maximum absolute error at convergence = {maximumAbsoluteError:.8e}")
    print(f"Maximum relative error at convergence = {maximumRelativeError:.8e}")

      
    return numberOfTerms

      
numberOfAnalyticalTerms = FindNumberOfTermsForConvergence(
    a=a,
    b=b,
    T0=TNorthWall,
    h=0.005,
    tolerance=1e-5
)

Terms:  10 ->  20 | Max. Absolute Error: 4.06046696e+00 | Max. Relative Error: 8.07775410e-02 | Worst Point: (0.005, 0.195)
Terms:  20 ->  30 | Max. Absolute Error: 3.53399268e-01 | Max. Relative Error: 5.53533269e-03 | Worst Point: (0.010, 0.195)
Terms:  30 ->  40 | Max. Absolute Error: 6.71461961e-02 | Max. Relative Error: 1.34498402e-03 | Worst Point: (0.195, 0.195)
Terms:  40 ->  50 | Max. Absolute Error: 8.32477700e-03 | Max. Relative Error: 1.33066086e-04 | Worst Point: (0.010, 0.195)
Terms:  50 ->  60 | Max. Absolute Error: 1.79884330e-03 | Max. Relative Error: 3.60259633e-05 | Worst Point: (0.005, 0.195)
Terms:  60 ->  70 | Max. Absolute Error: 2.47468314e-04 | Max. Relative Error: 3.98523645e-06 | Worst Point: (0.010, 0.195)
Terms:  70 ->  80 | Max. Absolute Error: 5.63475412e-05 | Max. Relative Error: 1.12849453e-06 | Worst Point: (0.005, 0.195)
Terms:  80 ->  90 | Max. Absolute Error: 8.15179155e-06 | Max. Relative Error: 1.31803978e-07 | Worst Point: (0.010, 0.195)

      
Analytical solution converged using 90 odd terms.
Highest value of n used = 179
Maximum absolute error at convergence = 8.15179155e-06
Maximum relative error at convergence = 1.31803978e-07

from scipy.interpolate import RegularGridInterpolator

      
fullLength = 2*a
fullWidth = b

      
comparisonPoints = [
    (fullLength/4, fullWidth/2),
    (fullLength/2, fullWidth/2),
    (3*fullLength/4, fullWidth/2),
    (fullLength/2, 3*fullWidth/4)
]

      
hValues = [0.02, 0.01, 0.005]
dumpPath = Path("../../Dumps/Problem1")

      
comparisonResults = []

      
for xPoint, yPoint in comparisonPoints:
    analyticalTemperature = AnalyticalSolution(xPoint, yPoint, fullLength, fullWidth, TNorthWall, numberOfAnalyticalTerms)
    rowResults = [xPoint, yPoint, analyticalTemperature]

      
    for h in hValues:
        temperatureValues = np.loadtxt(dumpPath/f"TemperatureSolution_h_{h:.5f}.csv", delimiter=",", skiprows=1, usecols=1)
        xHalf = np.arange(0, fullLength/2, h)
        yInterior = np.arange(h, fullWidth, h)
        temperatureHalf = temperatureValues.reshape(len(yInterior), len(xHalf))

      
        # Mirror the half-domain solution about x = a/2
        xRight = fullLength/2 + xHalf
        xInterior = np.concatenate((fullLength-xRight[:0:-1], xRight))
        temperatureFull = np.concatenate((temperatureHalf[:, :0:-1], temperatureHalf), axis=1)

      
        # Interpolate in case the requested point does not lie exactly on the mesh
        numericalInterpolator = RegularGridInterpolator((yInterior, xInterior), temperatureFull)
        numericalTemperature = float(numericalInterpolator((yPoint, xPoint)))
        absoluteError = abs(numericalTemperature-analyticalTemperature)
        rowResults.extend([numericalTemperature, absoluteError])

      
    comparisonResults.append(rowResults)

      
comparisonResults = np.array(comparisonResults)

      
print(f"{'x':>8}{'y':>8}{'Analytical':>14}{'h=0.020':>14}{'Error':>14}{'h=0.010':>14}{'Error':>14}{'h=0.005':>14}{'Error':>14}")
print("-"*114)

      
for row in comparisonResults:
    print(f"{row[0]:8.3f}{row[1]:8.3f}{row[2]:14.6f}{row[3]:14.6f}{row[4]:14.6e}{row[5]:14.6f}{row[6]:14.6e}{row[7]:14.6f}{row[8]:14.6e}")

x       y    Analytical       h=0.020         Error       h=0.010         Error       h=0.005         Error
------------------------------------------------------------------------------------------------------------------
   0.100   0.100     36.405666     36.373679  3.198750e-02     36.397647  8.019647e-03     36.403660  2.006283e-03
   0.200   0.100     44.511510     44.418976  9.253433e-02     44.488087  2.342332e-02     44.505636  5.873986e-03
   0.300   0.100     36.405666     36.373679  3.198750e-02     36.397647  8.019647e-03     36.403660  2.006283e-03
   0.200   0.150     70.995329     70.961193  3.413612e-02     70.972786  2.254262e-02     70.989692  5.636675e-03

def AnalyticalSolution(x, y, a, b, T0, numberOfTerms):
    temperature = 0.0
    for termId in range(numberOfTerms):
        n = 2*termId + 1
        sinhRatio = np.exp((n*np.pi/a)*(y-b))*(1-np.exp(-2*n*np.pi*y/a))/(1-np.exp(-2*n*np.pi*b/a))
        temperature += (1/n)*np.sin(n*np.pi*x/a)*sinhRatio
    return (4*T0/np.pi)*temperature

      
fullLength, fullWidth, halfLength, T0 = 0.4, 0.2, 0.2, 100.0
hValues = [0.02, 0.01, 0.005]
dumpPath = Path("../../Dumps/Problem1")

      
fig, axes = plt.subplots(1, 3, figsize=(18, 5), constrained_layout=True)

      
for plotId, h in enumerate(hValues):
    numericalHalf = np.loadtxt(dumpPath / f"TemperatureSolution_h_{h:.5f}.csv", delimiter=",", skiprows=1, usecols=1)
    xHalf, yInterior = np.arange(0, halfLength, h), np.arange(h, fullWidth, h)
    THalf = numericalHalf.reshape(len(yInterior), len(xHalf))

      
    # Mirror the half-domain solution about x = 0.2 m to get the full interior field
    xRight = halfLength + xHalf
    xInterior = np.concatenate((fullLength - xRight[:0:-1], xRight))
    TNumerical = np.concatenate((THalf[:, :0:-1], THalf), axis=1)

      
    X, Y = np.meshgrid(xInterior, yInterior)

      
    # Compute analytical solution at the same interior points
    TAnalytical = np.zeros_like(TNumerical)
    for rowId in range(Y.shape[0]):
        for columnId in range(X.shape[1]):
            TAnalytical[rowId, columnId] = AnalyticalSolution(X[rowId, columnId], Y[rowId, columnId], fullLength, fullWidth, T0, numberOfAnalyticalTerms)

      
    absoluteError = np.abs(TNumerical - TAnalytical)

      
    heatMap = axes[plotId].pcolormesh(X, Y, absoluteError, shading="auto", cmap="coolwarm")
    contourLines = axes[plotId].contour(X, Y, absoluteError, colors="black", linewidths=0.4, levels=10)

      
    axes[plotId].set_title(f"h = {h:.3f} m\nMax Error = {np.max(absoluteError):.6e}")
    axes[plotId].set_xlabel("x [m]")
    axes[plotId].set_ylabel("y [m]")
    axes[plotId].set_aspect("equal")

      
fig.colorbar(heatMap, ax=axes, shrink=0.85, label="Absolute Error")
fig.suptitle("Absolute Error Between Numerical and Analytical Solutions", fontsize=14)
plt.show()